### Extended Detention Basin

# Inspection and Maintenance Checklist

Property Address: 								Property Owner:

Treatment Measure No.:  		     Date of Inspection:  		          Type of Inspection:    Monthly                    Pre-Wet Season

 After heavy runoff (1” or greater)                                                                               												     End of Wet Season

Inspector(s):  		  Other:

| Defect | Conditions When Maintenance Is Needed |  | Maintenance Needed? (Y/N) |  | Comments (Describe maintenance completed and if any needed maintenance was not conducted, note when it will be done.) | Results Expected When Maintenance Is Performed |
| --- | --- | --- | --- | --- | --- | --- |
| ## General |  |  |  |  |  |  |
| Trash & Debris | - Trash and debris accumulated in basin. - Visual evidence of dumping. |  |  |  |  | Trash and debris cleared from site and disposed of properly. |
| Poisonous Vegetation and noxious weeds | Poisonous or nuisance vegetation or noxious weeds, e.g., morning glory, English ivy, reed canary grass, Japanese knotweed, purple loosestrife, blackberry, Scotch broom, poison oak, stinging nettles, or devil’s club. |  |  |  |  | Use Integrated Pest Management techniques to control noxious weeds or invasive species. |
| Contaminants and Pollution | Any evidence of oil, gasoline, contaminants or other pollutants. |  |  |  |  | No contaminants or pollutants present. |
| Rodent Holes | If facility acts as a dam or berm, any evidence of rodent holes, or any evidence of water piping through dam or berm via rodent holes. |  |  |  |  | The design specifications are not compromised by holes. Any rodent control activities are in accordance with applicable laws and do not affect any protected species. |
| Insects | Insects such as wasps and hornets interfere with maintenance activities. |  |  |  |  | Insects do not interfere with maintenance activities. |
| Tree/Brush Growth and Hazard Trees | - Growth does not allow maintenance access or interferes with maintenance activity. - Dead, diseased, or dying trees. |  |  |  |  | - Trees do not hinder maintenance activities. - Remove hazard trees as approved by the City. (Use a certified Arborist to determine health of tree or removal requirements). |
| Drainage time | Standing water remains in basin more than five days. |  |  |  |  | Correct any circumstances that restrict the flow of water from the system. Restore drainage to design condition. If the problem cannot be corrected and problems with standing water recur, then mosquitoes should be controlled with larvicides, applied by a licensed pesticide applicator. |
| Outfall structure | Debris or silt build-up obstructs an outfall structure. |  |  |  |  | Remove debris and/or silt build-up and dispose of properly. |
| # Side Slopes |  |  |  |  |  |  |
| Erosion | - Eroded over 2 in. deep where cause of damage is still present or where there is potential for continued erosion. - Any erosion on a compacted berm embankment. |  |  |  |  | Cause of erosion is managed appropriately. Side slopes or berm are restored to design specifications, as needed. |
| # Storage Area |  |  |  |  |  |  |
| Sediment | Accumulated sediment >10% of designed basin depth or affects inletting or outletting condition of the facility. |  |  |  |  | Sediment cleaned out to designed basin shape and depth; basin reseeded if necessary to control erosion. Sediment disposed of properly. |
| Liner (If Applicable) | Liner is visible and has more than three 1/4-inch holes in it. |  |  |  |  | Liner repaired or replaced. Liner is fully covered. |
| Emergency Overflow/ Spillway and Berms |  |  |  |  |  |  |
| Settlement | Berm settlement 4 inches lower than the design elevation. |  |  |  |  | Dike is built back to the design elevation. |
| Tree Growth | Tree growth on berms or emergency spillway >4 ft in height or covering more than 10% of spillway. |  |  |  |  | - Trees should be removed. If root system is small (base less than 4 inches) the root system may be left in place. Otherwise the roots should be removed and the berm restored. - A civil engineer should be consulted for proper berm/spillway restoration. |
| Emergency Overflow/ Spillway | Rock is missing and soil is exposed at top of spillway or outside slope. |  |  |  |  | Rocks and pad depth are restored to design standards. |
| Debris Barriers (e.g., Trash Racks) |  |  |  |  |  |  |
| Trash and Debris | Trash or debris is plugging openings in the barrier. |  |  |  |  | Trash or debris is removed and disposed of properly. |
| Damaged/ Missing Bars | Bars are missing, loose, bent out of shape, or deteriorating due to excessive rust. |  |  |  |  | Bars are repaired or replaced to allow proper functioning of trash rack. |
| Inlet/Outlet Pipe | Debris barrier is missing or not attached to pipe. |  |  |  |  | Debris barrier is repaired or replaced to allow proper functioning of trash rack. |
| #### Fencing and Gates |  |  |  |  |  |  |
| Missing or broken parts | Any defect in or damage to the fence or gate that permits easy entry to a facility. |  |  |  |  | Fencing and gate are restored to design specifications. |
| Deteriorating Paint or Protective Coating | Part or parts that have a rusting or scaling condition that has affected structural adequacy. |  |  |  |  | Paint or protective coating is sufficient to protect structural adequacy of fence or gate. |
| Flow Duration Control Outlet (if included in design to meet Hydromodification Management Standard) [[==refer to any attachments with additional provisions==]] |  |  |  |  |  |  |
| Risers, orifices and screens | Any debris or clogging |  |  |  |  | Restore unobstructed flow through discharge structure; to meet original design; dispose of debris properly. |
| Miscellaneous |  |  |  |  |  |  |
| Miscellan- ous | Any condition not covered above that needs attention to restore extended detention basin to design conditions. |  |  |  |  | Meets the design specifications. |